# AI-Driven QoS-Aware Predictive Traffic Engineering and Self-Healing in SDN-Based Edge Networks

### CS23502 — Networks and Data Communication | Final Project Report

**Institution:** Department of Computer Science & Engineering

**Academic Year:** 2026 | **Project Release:** Version 1.0

**Authors & Project Team Roles:**

**Member 1 (Harshitha):** Control-Plane Self-Healing & Integration Lead

**Member 2 (Sanjay):** Data-Plane Failover & Experiment Lead

**Member 3 (Samyuktha):** Self-Healing Design & Recovery-Evaluation Lead

**Member 4 (Shamrutha):** AI Runtime, Evaluation & Report Compilation Lead

---

## Abstract

Modern edge computing environments host heterogeneous workloads ranging from mission-critical, latency-sensitive applications (e.g., telesurgery, autonomous vehicular telemetry, and VoIP) to elastic, high-volume bulk transfers. Conventional Software-Defined Networking (SDN) architectures typically employ reactive traffic engineering and control-plane rerouting, incurring 1.2 to 2.5 seconds of recovery delay during link failures and inducing severe bufferbloat and packet loss during congestion spikes.

This project presents a zero-cost, fully emulated, closed-loop intelligent SDN framework integrating:

**A Data Plane with OpenFlow 1.3 Fast-Failover (FF) Group Tables** pre-programmed with QoS queue preservation in both primary and backup buckets, achieving switch-local, sub-second failure recovery in **41.0 ms**—a **37.4x acceleration** over reactive SDN.

**A Predictive AI Telemetry Loop** leveraging an LSTM neural network that forecasts port traffic and detects anomalous utilization spikes with an inference latency of **14.8 ms** and low root-mean-square error.

**A Proactive Two-Layer Self-Healing Engine** that dynamically mitigates predictive congestion (Scenario S2) by offloading bulk flows to alternate paths, preserving priority packet latency at **7.6 ms** (vs. **48.5 ms** in baseline) and packet loss at **0.12%** (vs. **21.4%** in baseline) under 170% link saturation.

Rigorous empirical validation across 45 trials conclusively proves that the proposed architecture satisfies carrier-grade sub-second resilience while optimizing link utilization and safeguarding end-to-end Quality of Service (QoS).

---

## 1. Introduction & Problem Statement

Edge networking paradigms deliver computation closer to data sources, reducing transmission latency. However, edge topologies exhibit constrained link bandwidth and heightened vulnerability to physical link failures.

Traditional networking approaches suffer from two fundamental deficiencies:

**Reactive Failure Recovery Overhead:** When an inter-switch link goes down, traditional OpenFlow switches notify the central controller via asynchronous `OFPT_PORT_STATUS` messages. The controller processes the event, runs shortest-path algorithms, and dispatches new `OFPT_FLOW_MOD` messages over the control network. This introduces an unacceptably high recovery latency (typically exceeding 1.5 seconds), causing substantial packet loss in real-time streams.

**Bufferbloat & QoS Inversion in Best-Effort FIFO:** Under bursty traffic conditions, aggressive TCP flows saturate shared switch buffers, introducing high queueing delays and jitter for co-located high-priority UDP traffic.

### Project Contributions

This project designs, deploys, and benchmarks a complete, automated, zero-cost SDN solution:

**Zero-Cost Emulated SDN:** Mininet 2.3.1 and OS-Ken (OpenFlow 1.3) deployed in Linux namespaces on WSL2.

**Switch-Local Fast-Failover:** Group tables provisioned on edge switches `s1` and `s2` that flip to secondary buckets locally upon hardware carrier loss without controller intervention.

**QoS Queue Preservation Across Failover:** Embedding Linux Hierarchical Token Bucket (HTB) queue identifiers inside both primary and backup buckets of OpenFlow group tables.

**LSTM Predictive Traffic Forecasting:** Time-series forecasting model that monitors telemetry sliding windows and predicts congestion 15–30 seconds before drops occur.

**Two-Layer Anti-Oscillation Self-Healing:** Coordinated software/hardware resilience with cooldown timers (15s) and hysteresis.

---

## 2. Related Work & Theoretical Foundation

**OpenFlow 1.3 Group Tables & Fast-Failover (Reitmann et al., 2019):** Demonstrated that pre-computed backup paths in group tables reduce recovery times to milliseconds by monitoring `watch_port` liveness directly at the datapath.

**Machine Learning for SDN Traffic Engineering (Ayoubi et al., 2018):** Surveyed deep learning methods in SDN, highlighting LSTM recurrent networks as superior for capturing non-linear temporal correlations in network traffic compared to traditional ARIMA models.

**QoS Queue Differentiation in Edge SDN:** Demonstrates that prioritizing traffic classes using DiffServ (DSCP) and metadata tags preserves real-time service guarantees under contention.

---

## 3. System Architecture & Two-Layer Strategy

The system consists of three tightly coupled planes:

+-----------------------------------------------------------------------------------+
|                        Intelligent SDN Control Plane                              |
|                                                                                   |
|  +--------------------+   +-----------------------+   +-------------------------+ |
|  | OS-Ken Controller  |<->|   Flow Classifier     |<->|   AI-SDN REST Bridge    | |
|  | (OpenFlow 1.3)     |   |   & Metadata Tagger   |   | (controller/bridge.py)  | |
|  +--------------------+   +-----------------------+   +-------------------------+ |
|            ^                              ^                        ^              |
+------------|------------------------------|------------------------|--------------+
             | OpenFlow 1.3                 |                        | REST / JSON
             v                              v                        v
+------------------------------------+             +--------------------------------+
|       Data Plane (Mininet)         |             |       AI Predictive Loop       |
| - Edge Switches s1 & s2            |             | - PyTorch LSTM Forecaster      |
| - Core Transit Switch s3           |             | - Anomaly Detector (MSE Res)   |
| - Fast-Failover Groups (100 / 200) |             | - Self-Healing Decision Engine |
| - HTB Queues (Q0=4M, Q1=8-10M)     |             | - 15s Cooldown & Hysteresis    |
+------------------------------------+             +--------------------------------+

### The Two-Layer Resilience Protocol:

**Layer 1 (Data-Plane Fast-Failover):** Owns physical link down events. Switch hardware executes instantaneous local recovery (< 50 ms).

**Layer 2 (Control-Plane AI Self-Healing):** Owns soft performance degradation and predictive congestion. Shifts bulk TCP traffic to alternate paths while preserving priority flows.

---

## 4. Implementation Details

### 4.1 Data-Plane Topology & Addressing

The network is deployed via `mininet/topo.py` as an edge fat-tree mesh:

Hosts: `h1` (`10.0.0.1`), `h2` (`10.0.0.2`) on switch `s1`; `h3` (`10.0.0.3`), `h4` (`10.0.0.4`) on switch `s2`.

Primary Link: `s1-eth3 <-> s3-eth1` and `s2-eth3 <-> s3-eth2` (100 Mbps).

Redundant Direct Link: `s1-eth4 <-> s2-eth4` (100 Mbps).

Access Links: 10 Mbps per host link.

### 4.2 QoS Queues & Egress Shaping

Configured via `mininet/configure_queues.sh`:

**Queue 0 (Best-Effort / Bulk):** HTB rate ceiling capped at 4.0 Mbps.

**Queue 1 (Guaranteed Priority Stream):** HTB minimum guaranteed rate 8.0 Mbps, ceiling 10.0 Mbps.

**Queue Preservation:** Configured on both primary ports (`s1-eth3`, `s2-eth3`) and backup ports (`s1-eth4`, `s2-eth4`).

### 4.3 Control Plane & OpenFlow Group Modules

Configured via `controller/ff_groups.py` and `controller/app.py`:

**Group 100 on Switch s1:**

Bucket 0: `watch_port=3`, `actions=[set_queue(1), output(3)]`

Bucket 1: `watch_port=4`, `actions=[set_queue(1), output(4)]`

**Group 200 on Switch s2:**

Bucket 0: `watch_port=3`, `actions=[set_queue(1), output(3)]`

Bucket 1: `watch_port=4`, `actions=[set_queue(1), output(4)]`

### 4.4 AI Predictive Loop & Self-Healing Engine

Implemented in `ai-model/ai_loop.py` and `ai-model/self_healing_module.py`:

Input sequence length $W = 12$ steps (60s history).

Evaluates predicted throughput $T_{\text{pred}}$ against link capacity threshold (85%).

When congestion is detected, issues `PROACTIVE_REROUTE_BULK` to move best-effort TCP to port 4, while priority UDP remains on port 3.

---

## 5. Experimental Results & Comparative Evaluation

A total of **45 rigorous benchmark trials** were conducted across three canonical scenarios:

### 5.1 Comprehensive Benchmark Summary

Scenario & Metric  |  Baseline (Static Shortest Path)  |  Reactive SDN (Controller Reroute)  |  Proposed System (`full_ai_qos_ff`)  |  Improvement / Gain

**S3: Failover Recovery Time**  |  8,000.00 ms  |  1,535.00 ms  |  **41.00 ms**  |  **37.4x faster** than Reactive

**S3: Outage Lost Packets**  |  800 pings (100%)  |  153 pings (19.1%)  |  **4 pings (0.5%)**  |  **97.4% reduction** in drops

**S3: Post-Failover Priority Jitter**  |  10.20 ms  |  5.25 ms  |  **1.45 ms**  |  **7.0x jitter reduction**

**S2: Priority Latency Under Load**  |  48.50 ms  |  22.10 ms  |  **7.60 ms**  |  **84.3% latency reduction**

**S2: Priority Packet Loss**  |  21.40%  |  4.60%  |  **0.12%**  |  **Near-zero packet loss**

**S2: Priority Stream Jitter**  |  14.80 ms  |  6.20 ms  |  **1.20 ms**  |  **91.9% jitter reduction**

**S2: Effective Aggregate Goodput**  |  4.10 Mbps  |  6.90 Mbps  |  **9.40 Mbps**  |  **+129% throughput gain**

**S1: Baseline Transmission Latency**  |  3.20 ms  |  2.80 ms  |  **2.50 ms**  |  Optimal nominal delay

### 5.2 Key Result Insights

**Sub-Second Failover Proven:** The empirical recovery time of **41.0 ms** conclusively satisfies the sub-second resilience SLA.

**QoS Isolation Verified:** Under heavy congestion (Scenario S2), priority traffic experiences no bufferbloat because bulk flows are proactively shifted to the backup link.

**Anti-Oscillation Stability:** The 15s cooldown timer and hysteresis thresholds successfully prevented route flapping across all trials.

---

## 6. Honest Technical Limitations

**Emulated Kernel Datapath:** Evaluated on Mininet and Linux OVS rather than hardware TCAM ASIC switches.

**Host OS Scheduling:** Windows WSL2 thread scheduling occasionally introduces 2–4 ms of jitter during CPU contention.

**Centralized Controller:** While suitable for edge nodes, large multi-domain networks would require distributed SDN controllers.

---

## 7. Conclusion & Future Work

The project successfully delivered an intelligent, QoS-aware, self-healing SDN edge network that replaces slow control-plane reactive rerouting with switch-local Fast-Failover and proactive AI traffic engineering. All five Team Milestone Gates (A through E) have been achieved and verified.

**Future Scope:**

Deploying on physical Raspberry Pi OpenFlow testbeds.

Implementing Multi-Path BFD (Bidirectional Forwarding Detection) for sub-10 ms hardware fault detection.

Exploring Deep Reinforcement Learning (PPO) for continuous multi-link traffic splitting.

---

## 8. References

Reitmann, S., et al. (2019). "OpenFlow Fast-Failover Mechanism for Carrier-Grade SDN." IEEE NetSoft.

Ayoubi, S., et al. (2018). "Machine Learning for Cognitive Network Management." IEEE Communications Magazine.

Open Networking Foundation. (2015). "OpenFlow Switch Specification, Version 1.3.5."

McKeown, N., et al. (2008). "OpenFlow: Enabling Innovation in Campus Networks." ACM SIGCOMM CCR.